IBC-X20 high-speed 2D scanning platform

Product features - Equipped with a new 500,000 global exposure scan engine - Larger scanning window for easier reading - Adjustable angle, flexible and easy to use - Frequently used function buttons, easier to set up - 5 styles of prompt sound, soft and bright LED indicator - Rich interface, strong compatibility, support a variety of systems and cash registers

IBC-X20 technical data

| Physical parameters |  |
| --- | --- |
| Scan mode | Auto-sensing/continuous scanning |
| Communication interface | USB-HID keyboard, USB-COM virtual serial port, RS232 serial port (optional). |
| Interface type | USB TYPE-B |
| Prompt mode | Buzzer, LED light |
| How it is set up | Press the key to set and manually scan the setting code in turn |
| Housing material | ABS+PC |
| Cable standard | Straight line 1 5m |
| Exterior dimensions | 1 55 mm×150mm×75mm(产品);200 mm×1 5 5 mm×85mm(外包装) |
| Weight | Net weight 483g; Gross weight 678g |
| Performance parameters |  |
| Scan type | CMOS global exposure |
| Resolution | 800*600 |
| Light source | 617nm red LED |
| Scan angle | ± 60°, ±54°, 360° (left and right, front and back, turn). |
| Scan rate | 120 times/sec |
| Field of view | 48° (H) x 30° (V) |
| Printing contrast | Minimum 20%. |
| Decoding kind | 1D: UPC-A, UPC-E0, UPC-E1, EAN-8, EAN13, Code128, GS1-128, Code39, Code32, Code93, Code11, Interleaved 2 of 5, Industrial 25, Matrix 2 of 5, Standard 25, CodaBar, MSI-plessey, GS1 Databar 2D: QR Code, Data Matrix (DM), PDF417, Maxi Code, Aztec Code, ISSN, NEC-25, COMPOSITE, supports mobile screen reading |
| Minimum resolution | 5thousand |
| Electrical parameters |  |
| Input voltage | DC 5V±10% |
| Current | Working: 240 mAh; Standby: 190 mAh |
| Environment parameters |  |
| Temperature | 0°C-50°C (working); -20°C~+70°C (storage). |
| Humidity | 5%-95% non-condensing |

The above data is subject to change without notice