IBC-X21/ IBC-X21L High-speed 2D scanning platform

Features: - Equipped with a new global exposure scan engine - Larger scanning window for easier reading - Adjustable angle, flexible and easy to use - Commonly used function buttons, easier to set - 5 styles of beeps, soft and bright LED indicators - Rich interfaces, strong compatibility, support a variety of systems and cash registers

IBC-X21/ IBC-X21L technical parameters

| Physical parameters |  |
| --- | --- |
| Scan mode | Auto-sensing/continuous scanning |
| Communication interface | USB-HID keyboard, USB-COM virtual serial port, RS232 serial port (optional). |
| Interface type | USB TYPE-B |
| Prompt way | Buzzer, LED light |
| How it's set up | Press the button to set it, and manually scan the setting code in turn |
| Housing material | ABS+PC |
| Cable standards | Straight line 1.5m |
| Exterior dimensions | 155mm× 155mm× 82mm (product), 210mm× 160.5mm× 100mm (outer packing). |
| Weight | Net weight 464.7g, gross weight 664.2g |
| Performance parameters |  |
| The type of scan | CMOS global exposure |
| Resolution | 1280*800 |
| Light source | 617nm red LED |
| Scan angle | ±70°, ±54°, 360° (left and right, front and back, rotation). |
| Scan rate | 120 times/second |
| Field of view | 48° (H) x 30° (V) |
| Print contrast | 20% minimum |
| Decode the kind | 1D: UPC-A, UPC-E0, UPC-E1, EAN-8, EAN13, Code128, GS1-128, Code39, Code32, Code93, Code11, Interleaved 2 of 5, Industrial 25, Matrix 2 of 5, Standard 25, CodaBar, MSI-plessey, GS1 Databar 2D: QR Code, Data Matrix (DM), PDF417, Maxi Code, Aztec Code, ISSN, NEC-25, COMPOSITE, support mobile screen reading |
| Minimum resolution | 4mil |
| Electrical parameters |  |
| Input voltage | DC 5V±10% |
| Current | Operating: 240 mA, Standby: 190 mA |
| Environmental parameters |  |
| Temperature | 0°C-50°C(Working);-20°C~+70°C(Storage). |
| Humidity | 5%-95% non-condensing |

The above information is subject to change without prior notice