IBC-X7 high-speed 2D scanning platform

Product features - Equipped with a new 500,000 global exposure scan engine - Larger scanning window for easier reading - Adjustable angle, flexible and easy to use - Frequently used function buttons, easier to set up - 5 styles of prompt sound, soft and bright LED indicator - Rich interface, strong compatibility, support a variety of systems and cash registers

IBC-X7 technical parameters

| Physical parameters |  |
| --- | --- |
| Scan mode | Auto-sensing/continuous scanning |
| Communication interface | USB-HID keyboard, USB-COM virtual serial port, RS232 serial port |
| Interface type | USB TYPE-B |
| Prompt mode | Buzzer, LED light |
| How it is set up | Press the key to set and manually scan the setting code in turn |
| Housing material | ABS+PC |
| Cable standard | Straight line 1.35m |
| Exterior dimensions | 148mm×97mm×97mm(产品);1 70 mm×1 15 mm×1 15 mm(外包装) |
| Weight | Net weight 326g; Gross weight 336g |
| Performance parameters |  |
| Scan type | CMOS global exposure |
| Resolution | 800*600 |
| Light source | 617nm red LED |
| Scan angle | ± 60°, ±54°, 360° (left and right, front and back, turn). |
| Scan rate | 120 fps |
| Field of view | 48° (H) x 30° (V) |
| Printing contrast | Minimum 20%. |
| Decoding kind | 1D: EAN13, EAN 8, UPC-A, UPC-E0, UPC-E1, Code128, Code39, Code93, Interleaved 2 of 5, Industrial 25, Matrix 2 of 5, Code11, CodeBar, MSI, RSS-14, RSS- Ltd, RSS- Exp, Standard 2 of 5, Plessey, ChinaPost 25, GS1 Composite 2D: QR Code, Data Matrix (DM), PDF417, Han Xin, Micro PDF417, Micro QRCode, MaxiCode, Aztec Code, Code16K, support mobile screen reading |
| Minimum resolution | 4thousand |
| Electrical parameters |  |
| Input voltage | DC 5V±10% |
| Current | Working: 240 mAh; Standby: 190 mAh |
| Environment parameters |  |
| Temperature | 0°C-50°C (working); -20°C~+70°C (storage). |
| Humidity | 5%-95% non-condensing |

The above data is subject to change without notice